| Course: |  | URET Phase 2 | Section: | 1 | Introduction |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 10 | Introduction |
| Lesson 4 |  | Keyboard Messages |  |  |  |

Topics Covered: - New UU Message - 13 shortcuts for URET functions (photo collage of keyboard, CRD with typing)

Quit Options Next GoTo Back Section 1: Introduction Repeat

- Audio:	      Production Notes:

| For controllers who prefer to use the keyboard, some of the URET functions can be performed using the U-U keyboard messages. This lesson covers the thirteen messages you can enter using these keyboard shortcuts. | Photo collage. Add bullets in sync with audio |
| --- | --- |

Branching:

| Next | 20 | Back | inactive |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Build 2 | Section: | 2 | Messages for 4 Displays |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 20 | Raising and Lowering Displays |
| Lesson 4 |  | Keyboard Messages |  |  |  |

(Plans) - ACL, Departure List, GPD and Wind Grid can be raised and lowered using a UU message Raising and Lowering Displays: (GPD) (Aircraft List) (Departure List) (D-CRD)

Quit Options Next GoTo Back Section 2: Messages for 4 Displays Repeat

- Audio:	    Production Notes:

| This section covers shortcut messages that apply to the Aircraft list, Departure List, GPD, and Wind Grid. These displays can all be raised to the top of the display or lowered to the bottom of the display using the UU message, depending on their position. | All URET displays open, DL on top. D-CRD, animate typing UU. |
| --- | --- |
| Typically, this message brings the specified display to the top of the screen. If it is already at the top of the stack, the display is lowered to the bottom. | ACL moves to top |
| If a display was previously displayed, it will come to the top at the size and location it was previously. If it was not yet displayed, it will appear in the default size and location. |  |

Branching:

| Next | 30 | Back | 10 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Build 2 | Section: | 2 | Messages for 4 Displays |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 30 | How to Raise/Lower Displays |
| Lesson 4 |  | Keyboard Messages |  |  |  |

(Wind Grid) - ACL – UU - DL – UU D - GPD – UU G - Wind Grid–UU W To Raise or Lower Displays: (GPD) (Aircraft List) (Departure List) (D-CRD) Try It: Practice raising and lowering the URET displays. When you’re finished, press NEXT to continue.

Quit Options Next GoTo Back Section 2: Messages for 4 Displays Repeat

- Audio:	    Production Notes:

| To raise or lower the Aircraft List, type UU in the D-CRD and then press ENTER. | All URET displays open, DL on top. D-CRD. Add 1st bullet. |
| --- | --- |
| To raise and lower the other displays, use the messages shown, followed by ENTER. | Add bullets 2- 4. |
| Try it. Practice raising and lowering the URET displays. When you’re finished, press NEXT to continue. | Add Try It text. Free practice. |
| If trackball click anywhere or type anything other than the correct answers: |  |
| To raise or lower a display, type U-U and press ENTER for the Aircraft List or U-U, a space, then the letter that represents the desired display, and then press enter. Try again. | If done incorrectly, show Incorrect. Try again. In red text and play audio. |

Branching:

| Next | 40 | Back | 20 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Build 2 | Section: | 2 | Messages for 4 Displays |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 40 | Raising the Wind Grid with Specified Altitude |
| Lesson 4 |  | Keyboard Messages |  |  |  |

(Wind Grid) - Type UU W (3-digit altitude) - Press ENTER Try It: ## Raise the Wind Grid to the top of the display with an altitude of FL310. To Raise the Wind Grid with a Specified Altitude: (GPD) (Aircraft List) (Departure List) (D-CRD)

Quit Options Next GoTo Back Section 2: Messages for 4 Displays Repeat

- Audio:	    Production Notes:

| The Wind Grid can also be raised or displayed with a specified altitude. | All URET displays open, DL on top. D-CRD. |
| --- | --- |
| ## Try it. Raise the Wind Grid to the top of the display with an altitude of flight level three one zero. | Add bullets and Try It text. |
| ## If type UU W 310 in the CRD and press ENTER: | Correct answer. |
| Good! | Remove text from CRD and bring the Wind Grid to the top of the display with altitude of 310. Show Correct! In green text and play audio. |
| If trackball click anywhere or type anything other than the correct answers: |  |
| To raise the wind grid to the top of the display with a specific altitude, type U-U, space, W, space, then the three digit altitude, in this case three-one-zero, and then press enter. Try again. | If done incorrectly, show Incorrect. Try again. In red text and play audio. |

Branching:

| Next | 50 | Back | 30 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Build 2 | Section: | 2 | Messages for 4 Displays |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 50 | Exiting Menus and Templates |
| Lesson 4 |  | Keyboard Messages |  |  |  |

- Type UU X - Press ENTER To Exit All Menus and Templates: (Aircraft List) (D-CRD) Try It: Exit the menus. (Flight Plan template)

Quit Options Next GoTo Back Section 2: Messages for 4 Displays Repeat

- Audio:	    Production Notes:

| Sometimes you may have a couple of menus or templates open. Rather than clicking on each one, you can close them all at once using a UU message. | ACL and Flight Plan Template. D-CRD. |
| --- | --- |
| Try it. Exit the menus. | Add bullets and Try It text. |
| If type UU X and press ENTER: | Correct answer. |
| Good! | Remove text from CRD and remove the Flight Plan Template. Show Correct! In green text and play audio. |
| If trackball click anywhere or type anything other than the correct answers: |  |
| To close all menus and templates on the display, type U-U, space, X, then press ENTER. Try again. | If done incorrectly, show Incorrect. Try again. In red text and play audio. |

Branching:

| Next | 60 | Back | 40 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Build 2 | Section: | 3 | Messages for the GPD |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 60 | Show/Remove Show |
| Lesson 4 |  | Keyboard Messages |  |  |  |

- Type UU S (CID, ACID, Code) - Press ENTER Try It: Show the alerts in the sector for PDT3237, CID 511 To Show/Remove Show: (Plans) (GPD) (Aircraft List) (Departure List) (D-CRD)

Quit Options Next GoTo Back Section 3: Messages for the GPD Repeat

- Audio:	    Production Notes:

| Section three covers U-U shortcut messages for the GPD. You can perform a Show or Remove a Show for an aircraft on the GPD. | All URET displays, DL on top. D-CRD. |
| --- | --- |
| Try it. Show the alerts in the sector for Piedmont thirty-two thirty-seven. | Add bullets and Try It text. |
| If type UU S PDT3237 or UU S 511 or UU S 1336 and press ENTER: | Correct answer. |
| That’s right. If the GPD is not already at the top of the stack, it will be raised to the top. The display will now include the current plan for the selection, along with the displayed route with all alerts assigned to the sector. If Show had already been applied to the selection, then entering the U-U-S message will remove the current plan from the GPD. | Remove text from CRD and display the GPD with the aircraft entered. Show Correct! In green text and play audio. |
| If trackball click anywhere or type anything other than the correct answers: |  |
| To perform a Show, type U-U, space S, space, and the C-I-D, aircraft ID, or code – in this case U Piedmont thirty-two thirty-seven, then press ENTER. Try again. | If done incorrectly, show Incorrect. Try again. In red text and play audio. |

Branching:

| Next | 70 | Back | 50 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Build 2 | Section: | 3 | Messages for the GPD |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 70 | Show All/Remove Show All |
| Lesson 4 |  | Keyboard Messages |  |  |  |

- Type UU A (CID, ACID, Code) - Press ENTER Try It: Show all alerts both inside and outside the sector for BTA3053, CID 501. To Show All/ Remove Show All: (Plans) (GPD) (Aircraft List) (Departure List) (D-CRD)

Quit Options Next GoTo Back Section 3: Messages for the GPD Repeat

- Audio:	    Production Notes:

| You can perform a Show All or Remove a Show All for an aircraft on the GPD. | All URET displays, DL on top. D-CRD. |
| --- | --- |
| Try it. Show all alerts both inside and outside the sector for Jetlink thirty fifty-three. | Add Try It text and bullets. |
| If type UU A BTA3053 or UU A 501 or UU A 2120 and press ENTER: | Correct answer |
| Very good. If the GPD is not already at the top of the stack, it will be raised to the top. The display will now include the current plan for the selection, along with the displayed route with all alerts inside and outside the sector for the aircraft. If Show All had already been applied to the selection, then entering the UU A message will remove the current plan from the GPD. | If done correctly, remove text from CRD and display the GPD with the aircraft entered. Show Correct! In green text and play audio. |
| If trackball click anywhere or type anything other than the correct answers: |  |
| To perform a Show All, type U-U, space A, space, and the C-I-D, aircraft ID, or code – in this case, Jetlink thirty fifty-three - then press ENTER. Try again. | If done incorrectly, show Incorrect. Try again. In red text and play audio. |

Branching:

| Next | 80 | Back | 60 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Build 2 | Section: | 4 | Messages for the ACL and DL |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 80 | Introduction |
| Lesson 4 |  | Keyboard Messages |  |  |  |

- Add /Find an Entry - Apply/Remove Highlighting - Scrolling Messages for the ACL and DL: (Aircraft List) (Departure List) (D-CRD)

Quit Options Next GoTo Back Section 4: Messages for the ACL and DL Repeat

- Audio:	    Production Notes:

| There are three UU Messages that can be performed in either the Aircraft List or the Departure List: Add or Find an entry, Apply or Remove highlighting coding and Scrolling to the top or bottom of the list. Let’s take a look at each. | ACL and DL. D-CRD. Add bullets in sync with audio. |
| --- | --- |

Branching:

| Next | 90 | Back | 70 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Build 2 | Section: | 4 | Messages for the ACL and DL |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 90 | Add/Find an Entry |
| Lesson 4 |  | Keyboard Messages |  |  |  |

- UU (CID, ACID, Code) - Press ENTER Try It: Find BLR555, CID 434. To Add or Find an Entry: (Aircraft List) (Departure List) (D-CRD)

Quit Options Next GoTo Back Section 4: Messages for the ACL and DL Repeat

- Audio:	    Production Notes:

| You can Add or Find an entry in the Aircraft List or the Departure List. Either the Aircraft List or the Departure List will be displayed at the top of the stack, depending on which list contains the specified entry. If the entry is in both the Aircraft List and the Departure List, the Aircraft List will be displayed at the top. | All URET displays, DL on top. D-CRD. |
| --- | --- |
| Try it. Find Blueridge five fifty-five. | Add bullets and Try It text. |
| If type UU BLR555 or UU 434 or UU 3624 and press ENTER: | Correct answer. |
| That’s right. Blueridge five fifty-five is in the Aircraft List, so the Aircraft List pops to the top of the display with the entry highlighted. | Remove text from CRD and display the ACL with BLR555 highlighted in reverse video. Show Correct! in green text and play audio. |
| If trackball click anywhere or type anything other than the correct answers: |  |
| To find an entry, type U-U, space, the C-I-D, aircraft ID, or code – in this case for Blueridge five fifty-five, then press ENTER. Try again. | If done incorrectly, show Incorrect. Try again. in red text and play audio. |

Branching:

| Next | 100 | Back | 80 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Build 2 | Section: | 4 | Messages for the ACL and DL |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 100 | Apply/Remove Highlighting |
| Lesson 4 |  | Keyboard Messages |  |  |  |

- UU H (CID, ACID, Code) - Press ENTER Try It: Apply highlighting to USA1729, CID 266. To Apply/Remove Highlighting: (Aircraft List) (Departure List) (D-CRD)

Quit Options Next GoTo Back Section 4: Messages for the ACL and DL Repeat

- Audio:	    Production Notes:

| You can apply and remove highlighting in either the Aircraft List or the Departure List with the UU message. | All URET displays, DL on top. D-CRD. |
| --- | --- |
| Try it. Apply highlighting to U.S. Air seventeen twenty-nine. | Add bullets and Try It text. |
| If type UU H USA1729, UU H 266 or UU H 7016 and press ENTER: | Correct answers. |
| That’s right. U.S. Air seventeen twenty-nine is in the Aircraft List, so the Aircraft List pops to the top of the display with the entry highlighted. If the entry is in both the Aircraft List and the Departure List, the Aircraft List will be displayed at the top with the entry highlighted. | Remove text from CRD and display the ACL with USA1729 highlighted in gray at top of list. Show Correct! in green text and play audio. |
| If trackball click anywhere or type anything other than the correct answers: |  |
| To apply highlighting, type U-U, space, the letter H, another space, the C-I-D, aircraft ID, or code – in this case U.S. Air seventeen twenty-nine - then press ENTER. Try again. | If done incorrectly, show Incorrect. Try again. in red text and play audio. |

Branching:

| Next | 110 | Back | 90 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Build 2 | Section: | 4 | Messages for the ACL and DL |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 110 | Scrolling |
| Lesson 4 |  | Keyboard Messages |  |  |  |

For ACL: - UU (Up or Down arrow) - Press ENTER For DL: - UU DL (Up or Down arrow) - Press ENTER To Scroll to the Top or Bottom of a Display: (Departure List) (Aircraft List) (D-CRD)

Quit Options Next GoTo Back Section 4: Messages for the ACL and DL Repeat

- Audio:	    Production Notes:

| You can use the UU message to scroll to the top or bottom of the Aircraft List or Departure List. The designated list will rise to the top of the display. If the Up arrow key is pressed, the list is scrolled so that the top entry in the list is shown as the first entry. If the down arrow key is pressed, then the list is scrolled so that the bottom entry is shown as the last entry. | ACL and DL. D-CRD. |
| --- | --- |
| Try it. Scroll to the bottom of the Departure List. | Add bullets and Try It text. |
| If type UU DL (Down arrow) and press ENTER. | Correct answer. |
| That’s right. | Remove text from CRD and display the DL scrolled to bottom. Show Correct! in green text and play audio. |
| If trackball click anywhere or type anything other than the correct answers: |  |
| To scroll to the bottom of the Departure List, type U-U, space, D-L, another space, then press the Down arrow key and press ENTER. Try again. | If done incorrectly, show Incorrect. Try again. in red text and play audio. |

Branching:

| Next | 120 | Back | 100 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Build 2 | Section: | 5 | Messages for the ACL |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 120 | Two Messages |
| Lesson 4 |  | Keyboard Messages |  |  |  |

- Changing the Posting Mode - Changing the Sort Criteria Two Messages for the ACL: (Aircraft List) (D-CRD)

Quit Options Next GoTo Back Section 5: Messages for the ACL Repeat

- Audio:	    Production Notes:

| There are two UU message shortcuts that can be performed on just the Aircraft List: Changing the Aircraft List Posting Mode and Changing the Aircraft List Sort Criteria. Let’s look at each. | ACL and D-CRD. Add bullets in sync. |
| --- | --- |

Branching:

| Next | 130 | Back | 110 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Build 2 | Section: | 5 | Messages for the ACL. |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 130 | Posting Mode |
| Lesson 4 |  | Keyboard Messages |  |  |  |

- UU P - Press ENTER - Posting Mode switches between Manual and Automatic Try It: Switch between Manual and Automatic Posting Modes. When you’re finished, click NEXT. To Change the Posting Mode: (Aircraft List) (D-CRD)

Quit Options Next GoTo Back Section 5: Messages for the ACL Repeat

- Audio:	    Production Notes:

| The U-U-P message allows you to change the Aircraft List Posting Mode. | ACL and DL. D-CRD. Animate typing UU P. Add bullet 1 and 2 |
| --- | --- |
| If not already displayed on top, the Aircraft List will be raised to the top of the display. If the current posting mode is automatic, then the posting mode is changed to manual. If the current posting mode is manual, then it is changed to automatic. | Add bullet 3. |
| Try it. Switch between Manual and Automatic Posting Modes. When you’re finished, click NEXT. | Add try it text. |
|  | Free play. 1st time, ACL should pop to the top and switch posting mode. All other times, just switch posting mode. |
| If trackball click anywhere or type anything other than the correct answers: |  |
| To scroll change the ACL posting mode, type U-U, space P and press ENTER. Try again. | If done incorrectly, show Incorrect. Try again. in red text and play audio. |

Branching:

| Next | 140 | Back | 120 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Build 2 | Section: | 5 | Messages for the ACL. |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 140 | Sort Criteria |
| Lesson 4 |  | Keyboard Messages |  |  |  |

- UU (desired Sort Criteria) - Press ENTER To Change the Sort Criteria: O: Default – Predicted Time at Sector Boundary OP: Predicted Time at Sector Boundary OT: Conflict Time OC: Conflict Status OS: Sector Position OA: ACID OD: Destination

Quit Options Next GoTo Back Section 5: Messages for the ACL Repeat

- Audio:	    Production Notes:

| To change the sort criteria for Aircraft List entries, type UU, a space, and then one of the choices listed here. Then press enter. | List of Sort Criteria. Add bullets in sync. |
| --- | --- |
| If not already displayed on top, the Aircraft List is raised to the top of the display and changed to the desired Sort Criteria. |  |

Branching:

| Next | 150 | Back | 130 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Build 2 | Section: | 5 | Messages for the ACL |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 150 | Try It |
| Lesson 4 |  | Keyboard Messages |  |  |  |

Practice changing the Sort Criteria. When you’re finished, click NEXT. O: Default (Predicted Time at Sector Boundary) OP: Predicted Time at Sector Boundary OT: Conflict Time OC: Conflict Status OS: Sector Position OA: ACID OD: Destination Try It: (Aircraft List) (D-CRD)

Quit Options Next GoTo Back Section 5: Messages for the ACL Repeat

- Audio:	    Production Notes:

| Try it. Practice changing the Sort Criteria. When you’re finished, click NEXT. | ACL and D-CRD. |
| --- | --- |
|  | Free play. Can type UU (desired sort criteria) and press ENTER. ACL should switch to one of the 6 versions of Sort Criteria (1st two are the same). |
| If trackball click anywhere or type anything other than the correct answers: |  |
| To change the Aircraft List Sort Criteria, Type UU, a space, and then the code corresponding to the desired Sort Criteria. Try again. | If perform incorrectly, show Incorrect. Try again. in red text on the screen and play audio. |

Branching:

| Next | 160 | Back | 140 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Build 2 | Section: | 6 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 160 | Introduction |
| Lesson 4 |  | Keyboard Messages |  |  |  |

(photo of URET) # End of Lesson Exercises

Quit Options Next GoTo Back Section 6: Exercises Repeat

- Audio:	    Production Notes:

| You may now begin the practice exercises. If you need help remembering which message to use for a function, click the Cheat Sheet button. | Photo of URET |
| --- | --- |

Branching:

| Next | 170 | Back | 150 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Build 2 | Section: | 6 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 170 | Exercise 1 |
| Lesson 4 |  | Keyboard Messages |  |  |  |

Exercise 1 of 12 Raise the Aircraft List to the top of the display. (Place Correct and Incorrect text feedback here when appropriate) (Plans) (GPD) (Aircraft List) (Departure List) (D-CRD) Cheat Sheet

Quit Options Next GoTo Back Section 6: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | All URET displays open, DL on top. D-CRD Button labeled Cheat Sheet should be provided on all screens. Graphic – the button is a sheet of legal paper with writing and a piece of tape at the top. |
| --- | --- |
| Correct! | Correct answer is to type: UU in the CRD and press ENTER. If done correctly, remove text from CRD and bring the Aircraft List to the top of the display. Show Correct! in green text and play audio. |
| Incorrect. To raise the Aircraft List to the top, type U-U and then press enter. Try again. | If done incorrectly, show Incorrect. Try again. in red text and play audio. |

Branching:

| Next | 180 | Back | 160 | Cheat Sheet | 175 |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Build 2 | Section: | 6 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 175 | Cheat Sheet |
| Lesson 4 |  | Keyboard Messages |  |  |  |

Exercise ? of 12 Leave Question Click RETURN to return to the exercise. Return ## Message; Purpose. UU; Raise/Lower ACL. UU D; Raise/Lower DL. UU G; Raise/Lower GPD. UU W; Raise/Lower Wind Grid. UU W ddd; Display/Raise Wind Grid with specified altitude, where ddd is a three digit altitude.. UU X; Exit/Close all menus and templates. UU S (CID/ACID/Code); Show/Remove Show the display of a current plan on the GPD.. UU A (CID/ACID/Code); Show All/Remove Show All the display of a current plan on the GPD.. UU (CID/ACID/Code); Add/Find an entry on the ACL or DL. UU H (CID/ACID/Code); Apply/Remove highlighting. UU (up or down arrow); Scroll to top or bottom of ACL. UU DL (up or down arrow); Scroll to top or bottom of DL. UU P; Change ACL Posting Mode. UU (desired Sort Criteria); Change Sort Criteria for ACL O: Default(predicted time at sector boundary) OP: predicted time at sector boundary OT: conflict time OC: conflict status OS: sector position OA: ACID OD: destination.

Quit Options Next GoTo Back Section 6: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | Provide this table whenever the student clicks the Cheat Sheet button throughout the exercises. Disable all navigation buttons. Change Cheat Sheet button to RETURN. Clicking this should return to the exercise. |
| --- | --- |

Branching:

| Next | inactive | Back | inactive | Return | Back to exercise |
| --- | --- | --- | --- | --- | --- |

Cheat Sheet

| Course: |  | URET Build 2 | Section: | 6 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 180 | Exercise 2 |
| Lesson 4 |  | Keyboard Messages |  |  |  |

Exercise 2 of 12 Lower the Departure List to the bottom of the display. (Place Correct and Incorrect text feedback here when appropriate) (Plans) (GPD) (Aircraft List) (Departure List) (D-CRD)

Quit Options Next GoTo Back Section 6: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | All URET displays open, DL on top. D-CRD Button labeled CHEAT SHEET should be provided on all screens. |
| --- | --- |
| Correct! | Correct answer is to type: UU D in the CRD and press ENTER. If done correctly, remove text from CRD and lower the Departure List to the bottom of the display. Show Correct! in green text and play audio. |
| Incorrect. To lower the Departure List to the bottom, type U-U, a space, D, and then press enter. Try again. | If done incorrectly, show Incorrect. Try again. in red text and play audio. |

Branching:

| Next | 190 | Back | 170 | CHEAT SHEET | 175 |
| --- | --- | --- | --- | --- | --- |

Cheat Sheet

| Course: |  | URET Build 2 | Section: | 6 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 190 | Exercise 3 |
| Lesson 4 |  | Keyboard Messages |  |  |  |

Exercise 3 of 12 Raise the Wind Grid to display winds at an altitude of FL350. (Place Correct and Incorrect text feedback here when appropriate) (Plans) (GPD) (Aircraft List) (Departure List) (D-CRD)

Quit Options Next GoTo Back Section 6: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | All URET displays open, DL on top. D-CRD Button labeled CHEAT SHEET should be provided on all screens. |
| --- | --- |
| Good! | Correct answer is to type: UU W 350 in the CRD and press ENTER. If done correctly, remove text from CRD and display the Wind Grid with an altitude of 350. Show Correct! in green text and play audio. |
| Incorrect. To display the Wind Grid with an altitude of flight level three-five-zero, type U-U, a space, W, space, three five zero, and then press enter. Try again. | If done incorrectly, show Incorrect. Try again. in red text and play audio. |

Branching:

| Next | 200 | Back | 180 | CHEAT SHEET | 175 |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Build 2 | Section: | 6 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 200 | Exercise 4 |
| Lesson 4 |  | Keyboard Messages |  |  |  |

Exercise 4 of 12 On the GPD, show the current plan and any alerts in the sector for PDT3237, CID 511. (Place Correct and Incorrect text feedback here when appropriate) (GPD) (D-CRD) Cheat Sheet

Quit Options Next GoTo Back Section 6: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | GPD and D-CRD Button labeled CHEAT SHEET should be provided on all screens. |
| --- | --- |
| That’s right! | Correct answer is to type: UU S PDT3237 or UU S 511 or UU S 1336 in the CRD and press ENTER. If done correctly, remove text from CRD and display the current plan on the GPD. Show Correct! in green text and play audio. |
| Incorrect. To show the current plan and any alerts in the sector, type U-U, a space, S, space, the C-I-D, Aircraft ID, or Code and then press enter. Try again. | If done incorrectly, show Incorrect. Try again. in red text and play audio. |

Branching:

| Next | 210 | Back | 190 | CHEAT SHEET | 175 |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Build 2 | Section: | 6 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 210 | Exercise 5 |
| Lesson 4 |  | Keyboard Messages |  |  |  |

Exercise 5 of 12 Remove the Show All for BTA3053, CID 501. (Place Correct and Incorrect text feedback here when appropriate) (GPD) (D-CRD) Cheat Sheet

Quit Options Next GoTo Back Section 6: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | GPD with a current plan for BTA3053 and D-CRD Button labeled CHEAT SHEET should be provided on all screens. |
| --- | --- |
| Correct! | Correct answer is to type: UU A BTA3053 or UU A 501, or UU A 2120 in the CRD and press ENTER. If done correctly, remove text from CRD and remove the current plan and alerts from the GPD Show Correct! in green text and play audio. |
| Incorrect. To remove the Show All, type U-U, a space, A, space, the C-I-D, Aircraft ID, or Code, and then press enter. Try again. | If done incorrectly, show Incorrect. Try again. in red text and play audio. |

Branching:

| Next | 220 | Back | 200 | CHEAT SHEET | 175 |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Build 2 | Section: | 6 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 220 | Exercise 6 |
| Lesson 4 |  | Keyboard Messages |  |  |  |

Exercise 6 of 12 Find the entry for BTA3053, CID 501. (Place Correct and Incorrect text feedback here when appropriate) (ACL) (DL) (D-CRD) Cheat Sheet

Quit Options Next GoTo Back Section 6: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | ACL, DL, and D-CRD. DL on top. Button labeled CHEAT SHEET should be provided on all screens. |
| --- | --- |
| Correct. | Correct answer is: Type UU BTA3053 or UU 501 or UU 2120 and press ENTER. If done correctly, remove text from CRD and display the ACL with BTA3053 highlighted in reverse video. Show Correct! in green text and play audio. |
| Incorrect. To find an entry, type U-U, space, the C-I-D, Aircraft ID, or Code, then press ENTER. Try again. | If done incorrectly, show Incorrect. Try again. in red text and play audio. |

Branching:

| Next | 230 | Back | 210 | CHEAT SHEET | 175 |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Build 2 | Section: | 6 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 230 | Exercise 7 |
| Lesson 4 |  | Keyboard Messages |  |  |  |

Exercise 7 of 12 Apply highlighting to PDT3237, CID 511. (Place Correct and Incorrect text feedback here when appropriate) (ACL) (DL) (D-CRD) Cheat Sheet

Quit Options Next GoTo Back Section 6: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | ACL, DL, and D-CRD. DL on top. Button labeled CHEAT SHEET should be provided on all screens. |
| --- | --- |
| Correct. | Correct answer is: Type UU H PDT3237 or UU H 511 or UU H 1336 and press ENTER. If done correctly, remove text from CRD and display the ACL with PDT3237 highlighted in gray. Show Correct! in green text and play audio. |
| Incorrect. To apply highlighting to the entry, type U-U, space, H, space, the C-I-D, Aircraft ID, or Code, then press ENTER. Try again. | If done incorrectly, show Incorrect. Try again. in red text and play audio. |

Branching:

| Next | 240 | Back | 220 | CHEAT SHEET | 175 |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Build 2 | Section: | 6 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 240 | Exercise 8 |
| Lesson 4 |  | Keyboard Messages |  |  |  |

Exercise 8 of 12 Scroll to the top of the ACL. (Place Correct and Incorrect text feedback here when appropriate) (ACL) (D-CRD) Cheat Sheet

Quit Options Next GoTo Back Section 6: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | ACL and D-CRD. ACL is scrolled down. Button labeled CHEAT SHEET should be provided on all screens. |
| --- | --- |
| Correct. | Correct answer is: Type UU, space (up arrow) and press ENTER. If done correctly, remove text from CRD and scroll the ACL to the top. Show Correct! in green text and play audio. |
| Incorrect. To scroll to the top of the Aircraft List, type U-U, a space, press the up arrow, then press ENTER. Try again. | If done incorrectly, show Incorrect. Try again. in red text and play audio. |

Branching:

| Next | 250 | Back | 230 | CHEAT SHEET | 175 |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Build 2 | Section: | 6 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 250 | Exercise 9 |
| Lesson 4 |  | Keyboard Messages |  |  |  |

Exercise 9 of 12 Change the ACL Posting Mode to Automatic. (Place Correct and Incorrect text feedback here when appropriate) (ACL) (D-CRD) Cheat Sheet

Quit Options Next GoTo Back Section 6: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | ACL and D-CRD. ACL Posting Mode is Manual. Button labeled CHEAT SHEET should be provided on all screens. |
| --- | --- |
| Good. | Correct answer is: Type UU P and press ENTER. If done correctly, remove text from CRD and change the ACL to manual posting. Show Correct! in green text and play audio. |
| Incorrect. To change the posting mode, type U-U, a space, P, then press ENTER. Try again. | If done incorrectly, show Incorrect. Try again. in red text and play audio. |

Branching:

| Next | 260 | Back | 240 | CHEAT SHEET | 175 |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Build 2 | Section: | 6 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 260 | Exercise 10 |
| Lesson 4 |  | Keyboard Messages |  |  |  |

Exercise 10 of 12 Change the ACL Sort Criteria to Sector Position. (Place Correct and Incorrect text feedback here when appropriate) (ACL) (D-CRD) Cheat Sheet

Quit Options Next GoTo Back Section 6: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | ACL and D-CRD. ACL Sort criteria is Predicted Time at Sector Boundary. Button labeled CHEAT SHEET should be provided on all screens. |
| --- | --- |
| That’s right. | Correct answer is: Type UU OS and press ENTER. If done correctly, remove text from CRD and change the ACL to sort by Sector Position. Show Correct! in green text and play audio. |
| Incorrect. To change the Sort Criteria to Sector Position, type U-U, a space, O-S, then press ENTER. Try again. | If done incorrectly, show Incorrect. Try again. in red text and play audio. |

Branching:

| Next | 270 | Back | 250 | CHEAT SHEET | 175 |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Build 2 | Section: | 6 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 270 | Exercise 11 |
| Lesson 4 |  | Keyboard Messages |  |  |  |

Exercise 11 of 12 Change the ACL Sort Criteria to ACID. (Place Correct and Incorrect text feedback here when appropriate) (ACL) (D-CRD) Cheat Sheet

Quit Options Next GoTo Back Section 6: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | ACL and D-CRD. ACL Sort criteria is Sector Position. Button labeled CHEAT SHEET should be provided on all screens. |
| --- | --- |
| Correct. | Correct answer is: Type UU OA and press ENTER. If done correctly, remove text from CRD and change the ACL to sort by ACID. Show Correct! in green text and play audio. |
| Incorrect. To change the Sort Criteria to Aircraft ID, type U-U, a space, O-A, then press ENTER. Try again. | If done incorrectly, show Incorrect. Try again. in red text and play audio. |

Branching:

| Next | 280 | Back | 260 | CHEAT SHEET | 175 |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Build 2 | Section: | 6 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 280 | Exercise 12 |
| Lesson 4 |  | Keyboard Messages |  |  |  |

Exercise 12 of 12 Close all the menus and templates at the same time. (Place Correct and Incorrect text feedback here when appropriate) (ACL) (D-CRD) (menu) (template) Cheat Sheet

Quit Options Next GoTo Back Section 6: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | ACL, menu, flight plan template and D-CRD. Button labeled CHEAT SHEET should be provided on all screens. |
| --- | --- |
| Correct. | Correct answer is: Type UU X and press ENTER. If done correctly, remove text from CRD and remove the menu and template. Show Correct! in green text and play audio. |
| Incorrect. To close all menus and templates, type U-U, a space, X then press ENTER. Try again. | If done incorrectly, show Incorrect. Try again. in red text and play audio. |

Branching:

| Next | 290 | Back | 270 | CHEAT SHEET | 175 |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Build 2 | Section: | 6 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 290 | Summary |
| Lesson 4 |  | Keyboard Messages |  |  |  |

## Exercise; Correct 1st Try?. - Raise the ACL to the top of the display.. 2. Lower the DL to the bottom of the display.. - Raise the Wind Grid with a specified altitude.. - Show a current plan on the GPD.. - Remove a Show All.. 6. Find an entry.. 7. Apply highlighting.. 8. Scroll to the top of a display.. 9. Change the ACL Posting Mode.. 10. Change the ACL Sort Criteria to Sector Position.. 11. Change the ACL Sort Criteria to ACID.. 12. Close all menus and templates.. Click on an exercise to review the topic.

Quit Options Next GoTo Back Section 6: End of Lesson Exercises Repeat

- Audio:	    Production Notes:

| You have completed the end of lesson exercises. This chart indicates which exercises you completed correctly on the first try. If you wish to review any of the topics, click on a question. | Table of exercises. Clicking on an exercise name returns the CBI to the point in the lesson that covers the question. Touch area should cover entire row. |
| --- | --- |

Branching:

| Next | 300 | Back | 280 | If click Ex 1 | 30 |
| --- | --- | --- | --- | --- | --- |
| If click Ex 2 | 30 | If click Ex 3 | 40 | If click Ex 4 | 60 |
| If click Ex 5 | 70 | If click Ex 6 | 90 | If click Ex 7 | 100 |
| If click Ex 8 | 110 | If click Ex 9 | 130 | If click Ex 10 | 140 |
| If click Ex 11 | 140 | If click Ex 12 | 50 |  |  |

| Course: |  | URET Build 2 | Section: | 7 | Summary |
| --- | --- | --- | --- | --- | --- |
| Day 4 |  |  | Frame | 300 | Summary |
| Lesson 4 |  | Keyboard Messages |  |  |  |

Topics Covered: - New UU Message - 13 shortcuts for URET functions (photo collage of keyboard, CRD with typing)

Quit Options Next GoTo Back Section 7: Summary Repeat

- Audio:	    Production Notes:

| You have completed the lesson on U-U messages available for shortcuts for URET functions. | Photo collage. |
| --- | --- |

Branching:

| Next | EXIT | Back | 290 |  |  |
| --- | --- | --- | --- | --- | --- |