| Course: |  | URET Phase 2 | Section: | 1 | Introduction |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 10 | Introduction |
| Lesson 2 |  | The GPD |  |  |  |

Topics Covered: - Purpose and capabilities - Layout and features - Recentering the map - Setting map options - Changing the display time - Modifying data blocks and trial data blocks - Cleaning up the GPD (CU of a GPD data block) - 1 PDT3237 140C PIT 240

Quit Options Next GoTo Back Section 1: Introduction Repeat

- Audio:	      Production Notes:

| In this lesson, we’ll explore the Graphic Plan Display, or GPD. The lesson will discuss: the purpose and capabilities of the GPD, the layout and features of the display, recentering the map, setting map options, changing the display time, modifying data blocks, and cleaning up the GPD. | CU of data block. Add bullets in sync with audio |
| --- | --- |

Branching:

| Next | 20 | Back | inactive |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 1 | Introduction |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 20 | Purpose |
| Lesson 2 |  | The GPD |  |  |  |

(GPD) Purpose: - Graphically depicts predicted conflicts, Trial Plans, Current Plans, and surrounding traffic - Predicted track data, NOT Radar data

Quit Options Next GoTo Back Section 1: Introduction Repeat

- Audio:	    Production Notes:

| The GPD graphically depicts predicted conflicts, trial plans, current plans, and surrounding traffic. | Add bullet 1. |
| --- | --- |
| The display shows aircraft routes, points of violation, and airspace. While it resembles Radar, it IS NOT radar data. It shows predictions of aircraft flight information. | Add bullet 2. |

Branching:

| Next | 30 | Back | 10 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 2 | GPD Layout |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 30 | Top of the Display |
| Lesson 2 |  | The GPD |  |  |  |

(GPD) Top of the Display: - Displays Current or Future Time indicator - Most buttons operate same as in other displays

Quit Options Next GoTo Back Section 2: GPD Layout Repeat

- Audio:	    Production Notes:

| Let’s take a look at the components of the GPD. |  |
| --- | --- |
| Along with the display title, the title bar will indicate whether the GPD is showing current time, or future time. | Add bullet 1. |
| The Plan Options, Show, Show All, FP Message, and Host AM buttons operate the same as they do in the other displays. We will discuss the rest of the buttons in this lesson and future lessons. | Add bullet 2. |

Branching:

| Next | 40 | Back | 20 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 2 | GPD Layout |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 40 | Map Elements |
| Lesson 2 |  | The GPD |  |  |  |

(GPD) Map Elements: - Most elements the same as radar display

Quit Options Next GoTo Back Section 2: GPD Layout Repeat

- Audio:	    Production Notes:

| The map elements of the GPD are almost the same as a radar display. Just as with a radar display, you’ll see Center, Sector, special airspace and approach control boundaries, airport and fix indicators, and position symbols, leader lines, and data blocks. | Add bullet 1. |
| --- | --- |

Branching:

| Next | 50 | Back | 30 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 3 | Data Blocks |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 50 | Data Block Lines |
| Lesson 2 |  | The GPD |  |  |  |

- 1 PDT3237 140C PIT 240 0 1 2 3 4 Click on each line of the data block to learn more. ### Data Block Lines

Quit Options Next GoTo Back Section 3: Data Blocks Repeat

- Audio:	    Production Notes:

| While the GPD data blocks are very similar to radar data blocks, there are some differences. | CU of PDT3237 data block. |
| --- | --- |
| The GPD data block has five lines – Line zero through line four. | Add numbers |
| Click on each of the lines to learn more about it. | Add prompt. Each line should have rollover and selected state. Hot area should include data block line and number. Mark each line with a green check after it is viewed. Next is inactive until all viewed. |

Branching:

| Next | 110 | Back | 40 | Line 0 | 60 |
| --- | --- | --- | --- | --- | --- |
| Line 1 | 70 | Line 2 | 80 | Line 3 | 90 |
| Line 4 | 100 |  |  |  |  |

| Course: |  | URET Phase 2 | Section: | 3 | Data Blocks |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 60 | Line Zero |
| Lesson 2 |  | The GPD |  |  |  |

- 1 PDT3237 140C PIT 240 0 1 2 3 4 Click on each line of the data block to learn more. ### Line 0

Quit Options Next GoTo Back Section 3: Data Blocks Repeat

- Audio:	    Production Notes:

| Line zero displays the alert indicators for the aircraft. The alert coding is the same as for other displays. | CU of PDT3237 data block with Line Zero highlighted with a yellow box. |
| --- | --- |
| If an aircraft is not being probed by URET, three brown Xs will appear in line zero. | Change the top line to brown XXX. |
| If there is nothing displayed in line zero, there are no alerts for the aircraft. | Make top line blank, rest of data block is white. |

Branching:

| Next | inactive | Back | 40 | Line 0 | 60 |
| --- | --- | --- | --- | --- | --- |
| Line 1 | 70 | Line 2 | 80 | Line 3 | 90 |
| Line 4 | 100 |  |  |  |  |

| Course: |  | URET Phase 2 | Section: | 3 | Data Blocks |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 70 | Line One |
| Lesson 2 |  | The GPD |  |  |  |

- 1 PDT3237 140C PIT 240 0 1 2 3 4 Click on each line of the data block to learn more. ### Line 1

Quit Options Next GoTo Back Section 3: Data Blocks Repeat

- Audio:	    Production Notes:

| Line one is the aircraft ID. | CU of PDT3237 data block with Line One highlighted with a yellow box. |
| --- | --- |

Branching:

| Next | inactive | Back | inactive | Line 0 | 60 |
| --- | --- | --- | --- | --- | --- |
| Line 1 | 70 | Line 2 | 80 | Line 3 | 90 |
| Line 4 | 100 |  |  |  |  |

| Course: |  | URET Phase 2 | Section: | 3 | Data Blocks |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 80 | Line Two |
| Lesson 2 |  | The GPD |  |  |  |

- 1 PDT3237 140C PIT 240 0 1 2 3 4 Click on each line of the data block to learn more. ### Line 2

Quit Options Next GoTo Back Section 3: Data Blocks Repeat

- Audio:	    Production Notes:

| Line two displays altitude data. Unlike radar, URET altitude is NOT Mode C. Rather, it is a trajectory based altitude and may differ from Mode C as much as fourteen hundred feet. | CU of PDT3237 data block with Line Two highlighted with a yellow box. |
| --- | --- |
| The altitude will be yellow to indicate I-AF-DOF or a block altitude. | Make the altitude yellow. |

Branching:

| Next | inactive | Back | inactive | Line 0 | 60 |
| --- | --- | --- | --- | --- | --- |
| Line 1 | 70 | Line 2 | 80 | Line 3 | 90 |
| Line 4 | 100 |  |  |  |  |

| Course: |  | URET Phase 2 | Section: | 3 | Data Blocks |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 90 | Line Three |
| Lesson 2 |  | The GPD |  |  |  |

- 1 PDT3237 140C PIT 240 0 1 2 3 4 Click on each line of the data block to learn more. ### Line 3

Quit Options Next GoTo Back Section 3: Data Blocks Repeat

- Audio:	    Production Notes:

| Line three displays | CU of PDT3237 data block with Line Three highlighted with a yellow box. |
| --- | --- |
| the destination, | Highlight just PIT |
| And the ground speed, as predicted by the trajectory. | Highlight just 240 |
| A blue box around the destination indicates that an ATC preferred route and/or HERT coding is applied to the entry. | Put blue box around PIT and add a yellow arrow with label: Indicates ATC Preferred Route and/or HERT coding. |

Branching:

| Next | inactive | Back | inactive | Line 0 | 60 |
| --- | --- | --- | --- | --- | --- |
| Line 1 | 70 | Line 2 | 80 | Line 3 | 90 |
| Line 4 | 100 |  |  |  |  |

| Course: |  | URET Phase 2 | Section: | 3 | Data Blocks |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 100 | Line Four |
| Lesson 2 |  | The GPD |  |  |  |

- 1 PDT3237 140C PIT 240 0 1 2 3 4 Click on each line of the data block to learn more. ### Line 4

Quit Options Next GoTo Back Section 3: Data Blocks Repeat

- Audio:	    Production Notes:

| Line four, when it is displayed, includes additional information, such as: | CU of PDT3237 data block with Line Four highlighted with a yellow box. |
| --- | --- |
| An aircraft’s requested altitude if the aircraft is climbing and an interim altitude is assigned; | Change line 2: 130T113 Change line 4: 140 (left justified) |
| The block altitude, if the aircraft is climbing or descending to the block altitude; | Change line 2: 140T113 Change line 4: 120B140 |
| Or a coordination fix time, if the aircraft’s origin is unknown and the coordination fix is in the future. | Change line 2: 140C Change line 4: E2120 |

Branching:

| Next | inactive | Back | inactive | Line 0 | 60 |
| --- | --- | --- | --- | --- | --- |
| Line 1 | 70 | Line 2 | 80 | Line 3 | 90 |
| Line 4 | 100 |  |  |  |  |

| Course: |  | URET Phase 2 | Section: | 3 | Data Blocks |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 110 | Colors |
| Lesson 2 |  | The GPD |  |  |  |

- 1 PDT3237 140C PIT 240 Colors: - Color of data block, position symbol, and leader line indicate highest alert - Green indicates no conflict 0 1 2 3 4

Quit Options Next GoTo Back Section 3: Data Blocks Repeat

- Audio:	    Production Notes:

| If the route line is displayed, the color of the data block, position symbol, and leader line indicates the highest conflict status of the aircraft. For example, if an aircraft has a red alert, the data block, position symbol, and leader line will all be red. | CU of red PDT3237 data block with leader line. |
| --- | --- |
| Other alert indicators will remain the color of that alert. | Highlight yellow 1. |
| If a data block is green, there is no conflict detected for the aircraft. | Change data block, position symbol and leader line to green. |
| If the route line is not displayed, the data block, position symbol, and leader line will be white, while the alert indicators will be the color of the alert type. | Remove leader line. Change data block to white. Leave 1s red and yellow |

Branching:

| Next | 120 | Back | 50 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 4 | Route of Flight |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 120 | Route Lines |
| Lesson 2 |  | The GPD |  |  |  |

Route Lines: - Same color as data block, position symbol, and leader line - Arrows next to route line indicate direction of flight (GPD)

Quit Options Next GoTo Back Section 4: Route of Flight Repeat

- Audio:	    Production Notes:

| Each aircraft on the GPD can also display a projected route of flight. The lines for the projected route of flight appear in the same color as the data block, position symbol and leader line. | GPD. Yellow arrow points to route line. |
| --- | --- |
| If there is an alert, the direction of flight is indicated by an arrow next to the route line with the arrowhead placed at the initial point of violation. This arrow is also colored to match the line. | Yellow arrow points to direction of flight arrows |

Branching:

| Next | 130 | Back | 110 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 4 | Route of Flight |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 130 | MSP & MEP |
| Lesson 2 |  | The GPD |  |  |  |

MSP & MEP: - MSP - Maneuver Start Point - MEP - Maneuver End Point (GPD)

Quit Options Next GoTo Back Section 4: Route of Flight Repeat

- Audio:	    Production Notes:

| The route line can also display a Maneuver Start Point, or M-S-P, and a Maneuver End Point, or M-E-P. | GPD |
| --- | --- |
| The M-S-P represents the point where URET predicts an altitude transition to begin. It is indicated with a white asterisk on the route line and an up or down arrow next to the altitude to which the aircraft is transitioning. | Highlight *(down arrow)110 |
| The M-E-P is the point where URET predicts the transition will be complete and is indicated with an asterisk and the altitude. | Highlight *110 |

Branching:

| Next | 140 | Back | 120 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 5 | Select a New Map Center Point |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 140 | Select a New Map Center Point |
| Lesson 2 |  | The GPD |  |  |  |

To Select a New Map Center Point: - Left click on Recenter - Left click on a new geographical point on the GPD. Try It: Create a new map center point on the Elkins airport symbol. (GPD) ELKINS

Quit Options Next GoTo Back Section 5: Select New Map Center Point Repeat

- Audio:	    Production Notes:

| The Recenter button allows you to select a new map center point. | GPD with Elkins airport labeled. |
| --- | --- |
| Try it. Create a new map center point on the Elkins airport symbol. | Add bullets and Try it text. |
| 1st step: left click Recenter button. | If done correctly, change cursor to a thick white X and put a big white plus in the center of the screen, as shown below. |
| Good. Notice that the cursor changed to an X and the current center point is indicated with a large plus sign. Now select the new center point. | Correct feedback. Highlight plus sign. |
| If middle or right click or left click anywhere else by on Recenter: |  |
| No, the first step is to left click on the Recenter button. Try again. | Incorrect feedback. Highlight Recenter button |
| 2nd step: left click on ELKINS or the square to the left of it. |  |
| Very good. | Correct feedback. Change to new GPD graphic that has ELKINS as center. |
| If middle or right click anywhere or left click anywhere else but on ELKINS: |  |
| For this exercise, please left click on the Elkins airport symbol for the new center point. Try again. | Incorrect feedback. Highlight ELKINS and the square next to it. |

Branching:

| Next | 150 | Back | 130 |  |  |
| --- | --- | --- | --- | --- | --- |

(GPD)

ELKINS

X

| Course: |  | URET Phase 2 | Section: | 6 | Range & Scale |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 150 | Change the Range |
| Lesson 2 |  | The GPD |  |  |  |

To Change the Range: - Left click on Range to decrease - Middle click on Range to increase Try It: Increase the Range by one step. (GPD) ELKINS

Quit Options Next GoTo Back Section 6: Range and Scale Repeat

- Audio:	    Production Notes:

| Use the Range button to change the range of the GPD map. | GPD with Elkins airport labeled. |
| --- | --- |
| Try it. Increase the map range by one step. | Add bullets and Try it text. |
| Correct answer: middle click Range button. | If done correctly, change to a new GPD graphic zoomed out and change the scale to so it has wider spaces. |
| Good. Notice that the map zoomed to a larger range. Also note that the scale has changed. | Correct feedback. Highlight scale. |
| If left or right click on Range: |  |
| No, you need to MIDDLE click to increase the range. Try again. | Incorrect feedback. Highlight Range button. |
| If left, right or middle click anywhere but on Range: |  |
| No, you need to middle click on the Range button to increase the range. Try again. | Incorrect feedback. Highlight Range button. |

Branching:

| Next | 160 | Back | 140 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 7 | Map Options |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 160 | Map Features Menu |
| Lesson 2 |  | The GPD |  |  |  |

To Change Map Features Options: - Left click on Map Options - Left click on Map Features - Left click on the desired option - Left click on EXIT to close the menu Try It: Using the Map Features Menu, turn on the high sector boundaries. Map Options Menu Map Features AC Display Menu (Map Features Menu)

Quit Options Next GoTo Back Section 7: Map Options Repeat

- Audio:	    Production Notes:

| You can change the features of the map and the aircraft display through the Map Options Menu. | Start with Map Options Menu with Map Features highlighted in reverse video and Map Features Menu. |
| --- | --- |
| Use the Map Features Menu to show or remove sector, center, and approach control boundaries; special activity airspace labels and boundaries; jet and victor airways; airport symbols and labels; fix labels; and the GPD scale indicator from the GPD Map. | Highlight each section on Map Features Menu. |
| Try it. Using the Map Features menu, turn on the High sector boundaries. | Change graphic to GPD. Add bullets and Try It text. |
| 1st step: left click on Map Options… | If done correctly, add Map Options Menu to screen. |
| If middle or right click or left click anywhere but on Map Options…: |  |
| No. The first step is to left-click on the Map Options button. Try again. | Incorrect feedback. Highlight Map Options button. |
| 2nd step: left click on Map Features | Highlight Map Features in Reverse Video and add Map Features Menu to screen. |
| If middle or right click or left click anywhere but on Map Features: |  |
| No. You need to left click on the Map Features option. Try again. | Incorrect feedback. Yellow highlight on Map Features. |
| 3rd step: left click on High in the box marked Sector Boundaries | If done correctly, add boundary line with arrow pointing to it. |
| If middle or right click or left click anywhere but on High: |  |
| Incorrect. You need to left click on High in the Sector Boundaries portion of the menu. Try again. | Incorrect feedback. Yellow highlight around High. |
| 4th Step: left click on Exit |  |
| Very Good. | Remove menu. |
| If middle or right click or left click anywhere but on EXIT: |  |
| You need to left click on Exit to remove the menu. Try again. | Incorrect feedback. Yellow highlight around EXIT. |

Branching:

| Next | 170 | Back | 150 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 7 | Map Options |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 170 | Aircraft Display Menu |
| Lesson 2 |  | The GPD |  |  |  |

Aircraft Display Menu: - Aircraft List Filters: display aircraft on the ACL - Altitude Filter Limits: remove display of aircraft at specified altitude - Auto Data Block Offset: enables or inhibits automatic data block offset - MSP/MEP Labels: suppress/restore labels - Route Preview: displays route preview lines in 1, 2, or 3 minute lengths Map Options Menu Map Features AC Display Menu (Aircraft Display Menu)

Quit Options Next GoTo Back Section 7: Map Options Repeat

- Audio:	    Production Notes:

| The Aircraft Display Menu allows you to manage the display of data blocks on the GPD using altitude list and altitude filters. | Start with Map Options Menu with Aircraft Display Menu highlighted in reverse video and Aircraft Display Menu. |
| --- | --- |
| Aircraft list filters display on the GPD aircraft that are on the aircraft list. | Highlight Aircraft list filters. |
| Altitude filter limits remove the display of aircraft at specified altitudes. Use the Filter Above and Filter Below input boxes to indicate the altitudes to be filtered. | Highlight Altitude filter limits |
| If you click in the Filter Above input box and type an altitude, the display of aircraft above that altitude will be removed from the GPD. | Highlight Filter Above and box |
| If you enter a value in the Filter Below input box, the display of aircraft below that altitude will be removed from the GPD. | Highlight Filter Below and box |
| The system will automatically offset data blocks on the GPD to prevent overlap. You can enable or inhibit this action by clicking the Auto Data block offset box. | Highlight Auto Data block offset |
| You can also suppress and/or restore Maneuver Start and Stop points labels by clicking the MSP MEP labels box. When the labels are suppressed, the asterisk denoting the maneuver start and end points will remain on the GPD. | Highlight MSP/MEP Labels |
| Finally, you can select a display of a preview of an aircraft’s route of flight. You can choose a preview length of one, two or three minutes. | Highlight Route Preview – whole box |

Branching:

| Next | 180 | Back | 160 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 7 | Map Options |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 180 | Changing the Aircraft Display |
| Lesson 2 |  | The GPD |  |  |  |

(GPD) To Change Aircraft Display Options: - Left click on Map Options - Left click on AC Display Menu - Left click on the desired option - Left click on EXIT to close the menu Try It: Using the Aircraft Display Menu, turn on the two-minute route preview. Map Options Menu Map Features AC Display Menu (Aircraft Display Menu)

Quit Options Next GoTo Back Section 7: Map Options Repeat

- Audio:	    Production Notes:

| Try it. Using the Aircraft Display menu, turn on the two-minute route preview. | GPD, bullets and Try It text. |
| --- | --- |
| 1st step: left click on Map Options… | If done correctly, add Map Options Menu to screen. |
| If middle or right click or left click anywhere but on Map Options…: |  |
| No. The first step is to left-click on the Map Options button. Try again. | Incorrect feedback. Highlight Map Options button. |
| 2nd step: left click on AC Display Menu | Highlight AC Display Menu in Reverse Video and add Aircraft Display Menu to screen. |
| If middle or right click or left click anywhere but on AC Display Menu: |  |
| No. You need to left click on the A-C Display Menu option. Try again. | Incorrect feedback. Yellow highlight on AC Display Menu. |
| 3rd step: left click on the diamond in front of 2. | If done correctly, add preview lines from each position symbol. |
| If middle or right click or left click anywhere but on the 2 diamond: |  |
| Incorrect. You need to left click on the diamond next to the two. Try again. | Incorrect feedback. Yellow highlight around diamond next to 2. |
| 4th Step: left click on Exit |  |
| Very Good. | Remove menu. |
| If middle or right click or left click anywhere but on EXIT: |  |
| You need to left click on Exit to remove the menu. Try again. | Incorrect feedback. Yellow highlight around EXIT. |

Branching:

| Next | 190 | Back | 170 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 8 | Display Time |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 190 | Changing the Aircraft Display |
| Lesson 2 |  | The GPD |  |  |  |

(GPD) To Change the Display Time: - Left click and hold on the Future Time Slider - Drag the slider to the desired time increment - Release the trackball button Try It: Change the GPD display time to ten minutes into the future.

Quit Options Next GoTo Back Section 8: Display Time Repeat

- Audio:	    Production Notes:

| The future time slider bar allows you to change the GPD to display the situation at a future time, up to twenty minutes. | GPD. Yellow arrow pointing to Future Time Slider square (not the whole rectangle). |
| --- | --- |
| Try it. Change the GPD display time to ten minutes into the future. | bullets and Try It text. |
| 1st step: left click and hold on the square inside the rectangle | If done correctly, proceed to 2nd step |
| If middle or right click or left click anywhere but on the square: |  |
| No. The first step is to left-click and hold on the future time slider bar. Try again. | Incorrect feedback. Highlight square inside rectangle. |
| 2nd step: drag the square to 10 minutes and release the trackball button. The number on top of the rectangle should increase by 1 and move to the right with the square as it is moved to the right. | If done correctly, fill in the rectangle with solid white, put a thicker white border around the map part of the GPD, and move the aircraft further along their routes. |
| Good. Notice that the slider bar rests at the ten-minute mark and all aircraft are projected along their routes ten minutes into the future. Also notice, that when the GPD displays in a future time mode, a white border surrounds the perimeter of the GPD |  |
| and the header changes to Future Time. | Highlight header text |
| You can also change the display to a future display by left-clicking anywhere in the future time slider area to advance in one minute increments or middle click in the slider area to advance to that time increment. |  |
| If release the trackball button without moving it or release it on any number other than 10: |  |
| No. Drag the future time slider bar to the ten-minute increment. Try again. | Incorrect feedback. |

Branching:

| Next | 200 | Back | 180 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 9 | Changing Data Blocks |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 200 | Offsetting Data Blocks |
| Lesson 2 |  | The GPD |  |  |  |

(GPD) To Offset a Data Block: - Left click and hold over the data block, position symbol, leader line, projected route line, or trial data block - Move the white box that appears to the new position - Release the trackball button Try It: Offset PDT3237’s data block into the yellow square.

Quit Options Next GoTo Back Section 9: Changing Data Blocks Repeat

- Audio:	    Production Notes:

| Just as with the radar display, the GPD data blocks can be offset and the leader line lengths can be changed. | GPD – no projected route lines. Yellow outline of a square is shown above PDT3237. |
| --- | --- |
| Try it. Offset Piedmont thirty-two thirty-seven’s data block into the yellow square. | Add bullets and Try It text. |
| 1st step: left click and hold on PDT3237’s data block, position symbol, or leader line. | If done correctly, add a white outline of a square over the data block. |
| If middle or right click or left click anywhere else: |  |
| No. The first step is to left-click and hold on Piedmont thirty-two thirty seven’s data block, position symbol, or leader line. Try again. | Incorrect feedback. Highlight PDT3237’s data block, position symbol, and leader line. |
| 2nd step: drag the square to the yellow box and release the trackball. | If done correctly, move the data block to the new position. |
| That’s correct. | Correct feedback. |
| If release the trackball button without moving it or release anywhere besides the yellow square. |  |
| No. For this exercise, please move the data block to the yellow square. Try again. | Incorrect feedback. |

Branching:

| Next | 210 | Back | 190 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 9 | Changing Data Blocks |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 210 | Suppress/Restore All Data Blocks |
| Lesson 2 |  | The GPD |  |  |  |

(GPD) To Suppress or Restore All Data Blocks: - Left click on the Suppress Menu Bar Button Try It: Suppress all the data blocks on the GPD. Suppress

Quit Options Next GoTo Back Section 9: Changing Data Blocks Repeat

- Audio:	    Production Notes:

| The display of all data blocks on the GPD can be suppressed, except a selected data block and data blocks for aircraft involved in a displayed trial plan and current plans. | GPD – no projected route lines. CDN242 is green. |
| --- | --- |
| Try it. Suppress all the data blocks on the GPD. | Add bullets and Try It text. |
| 1st step: left click on the Suppress button. | If done correctly, remove all data blocks except green CDN242. Change Suppress button to Restore. |
| That’s right. Notice that Canadian two forty-two’s data block remains on the GPD because it shows a current plan. |  |
| Also notice that the Suppress button has changed to Restore. To restore the data blocks, left click on Restore. | Highlight Restore button. |
| If middle or right click or left click anywhere else: |  |
| No. You need to left-click on the Suppress button. Try again. | Incorrect feedback. Highlight Suppress button. |

Branching:

| Next | 220 | Back | 200 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 9 | Changing Data Blocks |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 220 | Suppress/Restore a Single Data Block |
| Lesson 2 |  | The GPD |  |  |  |

(GPD) To Suppress or Restore a Single Data Block: - Right click on the position symbol of the desired data block Try It: Suppress Canadian two forty-two’s data block.

Quit Options Next GoTo Back Section 9: Changing Data Blocks Repeat

- Audio:	    Production Notes:

| You can suppress or restore an individual data block, unless alerts have been detected for that aircraft. | GPD – no projected route lines. CDN242 is white. |
| --- | --- |
| Try it. Suppress Canadian two forty-two’s data block. | Add bullets and Try It text. |
| 1st step: right click the diamond in front of CDN242. | If done correctly, remove CDN242’s data block and leader line. The position symbol changes to a white square with a black diamond outline. |
| Correct. Notice that the data block no longer appears on the GPD, but the position symbol remains. You can restore the data block by right-clicking on the position symbol again. If an alert is detected for an aircraft with a suppressed data block, the data block is automatically restored on the GPD. |  |
| If left or middle click anywhere: |  |
| Incorrect. For this task, you need to use the RIGHT trackball button. Try again. | Incorrect feedback. |
| If right click anywhere but on CDN242’s position symbol: |  |
| No. Here is Canadian two forty-two’s position symbol. Try again. | Incorrect feedback. Yellow highlight circle around CDN242’s position symbol. |

Branching:

| Next | 230 | Back | 210 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 10 | Accessing Sub Menus |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 230 | Accessing Sub Menus |
| Lesson 2 |  | The GPD |  |  |  |

(GPD) To Access Sub Menus from the GPD: - Left click on the Speed to access the Speed Menu - Left click on the Destination to access the Route Menu - Middle click on the Destination to display/remove a Flight Data Readout Try It: Display each of the sub menus for BTA3053. When you’re done, click NEXT.

Quit Options Next GoTo Back Section 10: Accessing Sub Menus Repeat

- Audio:	    Production Notes:

| Clicking on the speed and destination in the data block will access the same sub menus as the ACL. | GPD – no projected route lines. Highlight CLE and 245. |
| --- | --- |
| Try it. Display the various sub menus for Jetlink thirty fifty-three. When you’re finished, click Next. | Add bullets and Try It text. |
| Can perform either of 3 tasks: | Must perform all three before NEXT is active |
| If left click on 245, add Speed menu as shown below. If left click on CLE, add Route Menu as shown below. If middle click on CLE, add Response Display as shown below |  |
| If right click anywhere: |  |
| For these tasks, use the left or middle trackball button. Try again. | Incorrect feedback. |
| If left click anywhere else but CLE or 245: |  |
| No. You need to click on either the destination or the speed. Try again. | Highlight CLE and 245. |
| If middle click anywhere else but CLE: |  |
| No. To display a Flight Data Readout, middle-click on the destination. Try again. | Highlight CLE. |

Branching:

| Next | 240 | Back | 220 |  |  |
| --- | --- | --- | --- | --- | --- |

BTA3053   B190/A

Direct-to-Fix

Include PAR

Append *Append

MITEREKN346010

CEROLCKB

DEKAYBURGS

EKNCLE

Route Menu

Amend

Trial Plan

GVE.V38.EKN.V37.CKB..CLE

Flight Data

Previous Route

Exit

(Response Display)

501 BTA3053 B190/A 2120 140 EKN E2130 180 RIC./.GVE.V38.EKN..CLE

(speed menu

180 is highlighted in reverse video)

| Course: |  | URET Phase 2 | Section: | 11 | Clean Up the GPD |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 240 | Clean Up the GPD |
| Lesson 2 |  | The GPD |  |  |  |

(GPD) To Clean Up the GPD: - Left- click on the Clean Up menu bar button Try It: Clean up the GPD.

Quit Options Next GoTo Back Section 11: Clean Up the GPD Repeat

- Audio:	    Production Notes:

| The Clean Up menu bar button allows you to remove the display of all route lines for the GPD. | GPD with route lines displayed. Highlight Clean Up button. |
| --- | --- |
| Try it. Clean up the GPD. | Add bullet and Try It text. |
| If left click on Clean Up: |  |
| Correct. | Correct feedback. Remove all route lines and change data blocks to white, except for alert numbers. |
| If right or middle click or left click anywhere else but Clean Up: |  |
| No. You need to left click on the Clean Up menu bar button. Try again. | Highlight Clean Up. |

Branching:

| Next | 250 | Back | 230 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 12 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 250 | Introduction |
| Lesson 2 |  | The GPD |  |  |  |

(photo of URET) # End of Lesson Exercises

Quit Options Next GoTo Back Section 12: Exercises Repeat

- Audio:	    Production Notes:

|  | Photo of URET |
| --- | --- |

Branching:

| Next | 260 | Back | 240 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 12 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 270 | Exercise 1 |
| Lesson 2 |  | The GPD |  |  |  |

Exercise 1 of 11 Click on PDT3237’s MSP. (Place Correct and Incorrect text feedback here when appropriate) (GPD) - 1 PDT3237 140C PIT 240 * 110

Quit Options Next GoTo Back Section 12: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | GPD with route lines displayed. |
| --- | --- |
| If click on the white asterisk, down arrow, or 110 next to the red line: |  |
| Correct. | Correct! in green text. Auto advance to next exercise. |
| If middle or right click anywhere or left click anywhere except the asterisk, etc.: |  |
| Incorrect. An asterisk, down arrow and altitude indicate the maneuver start point. Try again. | Show Incorrect. Try again. Highlight white asterisk, down arrow, and 110. |

Branching:

| Next | 280 | Back | 260 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 12 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 280 | Exercise 2 |
| Lesson 2 |  | The GPD |  |  |  |

Exercise 2 of 11 Recenter the map over the EVERS MOA. (Place Correct and Incorrect text feedback here when appropriate) (GPD) - 1 PDT3237 140C PIT 240 EVERS MOA

Quit Options Next GoTo Back Section 12: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | GPD with no route lines displayed. EVERS MOA is labeled. |
| --- | --- |
| 1st step: left click Recenter button. | If done correctly, change cursor to a thick white X and put a big white plus in the center of the screen, as in frame 140. |
| If middle or right click or left click anywhere else by on Recenter: | Change to new GPD graphic that has EVERS MOA as center. |
| No, the first step is to left click on the Recenter button. Try again. | Incorrect feedback. Highlight Recenter button |
| 2nd step: left click anywhere in or on the EVERS MOA square (not just the label) |  |
| If middle or right click anywhere or left click anywhere else but on EVERS MOA: |  |
| For this exercise, please left click on the EVERS MOA for the new center point. Try again. | Incorrect feedback. Highlight the large rectangle labeled EVERS MOA. |

Branching:

| Next | 290 | Back | 270 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 12 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 290 | Exercise 3 |
| Lesson 2 |  | The GPD |  |  |  |

Exercise 3 of 11 Increase the Range of the GPD by one step. (Place Correct and Incorrect text feedback here when appropriate) (GPD) - 1 PDT3237 140C PIT 240

Quit Options Next GoTo Back Section 12: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | GPD with no route lines displayed. |
| --- | --- |
| Correct answer: Middle click Range button. | If done correctly, change to a new GPD graphic zoomed out and change the scale. |
| Good. | Correct feedback. |
| If left, right or middle click anywhere but on Range: |  |
| No, you need to MIDDLE click on the Range button to increase the range. Try again. | Incorrect feedback. Highlight Range button. |

Branching:

| Next | 300 | Back | 280 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 12 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 300 | Exercise 4 |
| Lesson 2 |  | The GPD |  |  |  |

Exercise 4 of 11 Set the GPD to display fix labels. (Place Correct and Incorrect text feedback here when appropriate) (GPD) - 1 PDT3237 140C PIT 240

Quit Options Next GoTo Back Section 12: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | GPD with no route lines displayed. |
| --- | --- |
| 1st step: left click on Map Options… | If done correctly, add Map Options Menu to screen. |
| If middle or right click or left click anywhere but on Map Options…: |  |
| No. The first step is to left-click on the Map Options button. Try again. | Incorrect feedback. Highlight Map Options button. |
| 2nd step: left click on Map Features | Highlight Map Features in Reverse Video and add Map Features Menu to screen. |
| If middle or right click or left click anywhere but on Map Features: |  |
| No. You need to left click on the Map Features option. Try again. | Incorrect feedback. Yellow highlight on Map Features. |
| 3rd step: left click on Fix Labels at the bottom of the menu | If done correctly, label the + in the middle of the display BUGEL |
| If middle or right click or left click anywhere but on FIX LABELS: |  |
| Incorrect. You need to left click on Fix Labels portion of the menu. Try again. | Incorrect feedback. Yellow highlight around Fix Labels. |
| 4th Step: left click on Exit |  |
| Very Good. | Remove menu. |
| If middle or right click or left click anywhere but on EXIT: |  |
| You need to left click on Exit to remove the menu. Try again. | Incorrect feedback. Yellow highlight around EXIT. |

Branching:

| Next | 310 | Back | 290 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 12 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 310 | Exercise 5 |
| Lesson 2 |  | The GPD |  |  |  |

Exercise 5 of 11 Turn on the one minute route preview. (Place Correct and Incorrect text feedback here when appropriate) (GPD) - 1 PDT3237 140C PIT 240

Quit Options Next GoTo Back Section 12: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | GPD with no route lines displayed. |
| --- | --- |
| 1st step: left click on Map Options… | If done correctly, add Map Options Menu to screen. |
| If middle or right click or left click anywhere but on Map Options…: |  |
| No. The first step is to left-click on the Map Options button. Try again. | Incorrect feedback. Highlight Map Options button. |
| 2nd step: left click on AC Display Menu | Highlight AC Display Menu in Reverse Video and add Aircraft Display Menu to screen. |
| If middle or right click or left click anywhere but on AC Display Menu: |  |
| No. You need to left click on the A-C Display Menu option. Try again. | Incorrect feedback. Yellow highlight on AC Display Menu. |
| 3rd step: left click on the diamond in front of 1. | If done correctly, add preview lines from each position symbol. |
| If middle or right click or left click anywhere but on the 1 diamond: |  |
| Incorrect. You need to left click on the diamond next to the one. Try again. | Incorrect feedback. Yellow highlight around diamond next to 1. |
| 4th Step: left click on Exit |  |
| Very Good. | Remove menu. |
| If middle or right click or left click anywhere but on EXIT: |  |
| You need to left click on Exit to remove the menu. Try again. | Incorrect feedback. Yellow highlight around EXIT. |

Branching:

| Next | 320 | Back | 300 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 12 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 320 | Exercise 6 |
| Lesson 2 |  | The GPD |  |  |  |

Exercise 6 of 11 Change the GPD display time to five minutes into the future. (Place Correct and Incorrect text feedback here when appropriate) (GPD) - 1 PDT3237 140C PIT 240

Quit Options Next GoTo Back Section 12: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | GPD with no route lines displayed. |
| --- | --- |
| 1st step: left click and hold on the square inside the rectangle | If done correctly, proceed to 2nd step |
| That’s right. | Correct feedback. |
| If middle or right click or left click anywhere but on the square: |  |
| No. The first step is to left-click and hold on the future time slider bar. Try again. | Incorrect feedback. Highlight square inside rectangle. |
| 2nd step: drag the square to 5 minutes and release the trackball button. The number on top of the rectangle should increase by 1 and move to the right with the square as it is moved to the right. | If done correctly, fill in the rectangle with solid white, put a thicker white border around the map part of the GPD, and move the aircraft further along their routes. |
| If release the trackball button without moving it or release it on any number other than 5: |  |
| No. Drag the future time slider bar to the five-minute increment. Try again. | Incorrect feedback. |
| Allow Optional method – can left click anywhere in the slider bar five times. Map and numbers above slider increase by one increment at a time. |  |

Branching:

| Next | 330 | Back | 310 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 12 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 330 | Exercise 7 |
| Lesson 2 |  | The GPD |  |  |  |

Exercise 7 of 11 Offset BLR555’s data block to the yellow box. (Place Correct and Incorrect text feedback here when appropriate) (GPD) 1 BLR555 120C CRW 200

Quit Options Next GoTo Back Section 12: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | GPD with no route lines displayed. |
| --- | --- |
| 1st step: left click and hold on BLR555’s data block, position symbol, or leader line. | If done correctly, add a white outline of a square over the data block. |
| Very good. | Correct feedback. |
| If middle or right click or left click anywhere else: |  |
| No. The first step is to left-click and hold on Blueridge five fifty-five’s data block, position symbol, or leader line. Try again. | Incorrect feedback. Highlight BLR555’s data block, position symbol, and leader line. |
| 2nd step: drag the square to the yellow box and release the trackball. | If done correctly, move the data block to the new position. |
| That’s correct. | Correct feedback. |
| If release the trackball button without moving it or release anywhere besides the yellow square. |  |
| No. For this exercise, please move the data block to the yellow square. Try again. | Incorrect feedback. |

Branching:

| Next | 340 | Back | 320 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 12 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 340 | Exercise 8 |
| Lesson 2 |  | The GPD |  |  |  |

Exercise 8 of 11 Restore the display of data blocks to the GPD. (Place Correct and Incorrect text feedback here when appropriate) (GPD) Restore

Quit Options Next GoTo Back Section 12: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | GPD with only CDN242 displayed. |
| --- | --- |
| 1st step: left click on the Restore button. | If done correctly, display all the data blocks. Change Restore button to Suppress. |
| Good. | Correct feedback. |
| If middle or right click or left click anywhere else: |  |
| No. You need to left-click on the Restore button. Try again. | Incorrect feedback. Highlight Restore button. |

Branching:

| Next | 350 | Back | 330 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 12 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 350 | Exercise 9 |
| Lesson 2 |  | The GPD |  |  |  |

Exercise 9 of 11 Restore the suppressed data block. (Place Correct and Incorrect text feedback here when appropriate) (GPD)

Quit Options Next GoTo Back Section 12: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

| Make sure button says Suppress not Restore. | GPD. PDT3237’s data block is suppressed. Only a white box with a diamond shows. |
| --- | --- |
| 1st step: right click the diamond in the square: | If done correctly, put PDT3237’s data block back. |
| Correct. | Correct feedback |
| If left or middle click anywhere: |  |
| Incorrect. For this task, you need to use the RIGHT trackball button. Try again. | Incorrect feedback. |
| If right click anywhere but on the diamond in the square: |  |
| No. Here is the suppressed data block. Try again. | Incorrect feedback. Yellow highlight circle the diamond in the square |

Branching:

| Next | 360 | Back | 340 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 12 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 360 | Exercise 10 |
| Lesson 2 |  | The GPD |  |  |  |

Exercise 10 of 11 Display a flight data readout for PDT3237. (Place Correct and Incorrect text feedback here when appropriate) (GPD) - 1 PDT3237 140C PIT 240

Quit Options Next GoTo Back Section 12: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | GPD. PDT3237’s data block is suppressed. Only a white box with a diamond shows. |
| --- | --- |
| 1st step: middle click on PIT: | If done correctly, add the response display with a flight data readout. (need this) |
| Correct. | Correct feedback |
| If left or right click anywhere: |  |
| Incorrect. For this task, you need to use the MIDDLE trackball button. Try again. | Incorrect feedback. |
| If middle click anywhere but on PIT: |  |
| No. To display a flight data readout, middle click on the destination. Try again. | Incorrect feedback. Yellow highlight around PIT |

Branching:

| Next | 370 | Back | 350 |  |  |
| --- | --- | --- | --- | --- | --- |

(Response Display)

511 PDT3237 T/DH8A/I  1336 140 EKN E2150 180 CHO./.DEKAY.NESTO1.PIT

| Course: |  | URET Phase 2 | Section: | 12 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 370 | Exercise 11 |
| Lesson 2 |  | The GPD |  |  |  |

Exercise 11 of 11 Clean up the GPD. (Place Correct and Incorrect text feedback here when appropriate) (GPD) - 1 PDT3237 140C PIT 240

Quit Options Next GoTo Back Section 12: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | GPD with route lines displayed. |
| --- | --- |
| If left click on Clean Up: |  |
| Correct. | Correct feedback. Remove all route lines and change data blocks to white, except for alert numbers. |
| If right or middle click or left click anywhere else but Clean Up: |  |
| No. You need to left click on the Clean Up menu bar button. Try again. | Highlight Clean Up. |

Branching:

| Next | 380 | Back | 360 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 12 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 390 | Summary |
| Lesson 2 |  | The GPD |  |  |  |

## Exercise; Correct 1st Try?. 1. Identify the MSP.. - Select a new map center point.. - Change the range of the GPD.. - Use the Map Features Menu.. 5. Use the Aircraft Display Menu.. 6. Change the GPD display time.. 7. Offset a data block.. 8. Restore all data blocks to the GPD.. 9. Restore a single data block.. 10. Display a flight data readout.. 11. Clean up the GPD.. Click on an exercise to review the topic.

Quit Options Next GoTo Back Section 12: End of Lesson Exercises Repeat

- Audio:	    Production Notes:

|  | Table of exercises. Clicking on an exercise name returns the CBI to the point in the lesson that covers the question. Touch area should cover entire row. |
| --- | --- |

Branching:

| Next | 400 | Back | 380 | If click Ex 1 | 120 |
| --- | --- | --- | --- | --- | --- |
| If click Ex 2 | 130 | If click Ex 3 | 140 | If click Ex 4 | 150 |
| If click Ex 5 | 160 | If click Ex 6 | 180 | If click Ex 7 | 190 |
| If click Ex 8 | 200 | If click Ex 9 | 210 | If click Ex 10 | 220 |
| If click Ex 11 | 230 | If click Ex 12 | 240 |  |  |

| Course: |  | URET Phase 2 | Section: | 13 | Summary |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 400 | Summary |
| Lesson 2 |  | The GPD |  |  |  |

Topics Covered: - Purpose and capabilities - Layout and features - Recentering the map - Setting map options - Changing the display time - Modifying data blocks and trial data blocks - Cleaning up the GPD (CU of a GPD data block) - 1 PDT3237 140C PIT 240

Quit Options Next GoTo Back Section 13: Summary Repeat

- Audio:	    Production Notes:

| You have completed the lesson on the GPD. | CU of data block with bullets. |
| --- | --- |

Branching:

| Next | EXIT | Back | 390 |  |  |
| --- | --- | --- | --- | --- | --- |