| Course: |  | URET Phase 2 | Section: | 1 | Introduction |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 10 | Introduction |
| Lesson 6 |  | More Trial Planning Features |  |  |  |

Topics Covered: - Invalid Trial Plans - Resubmitting a Trial Plan - Sending an Amendment - Sending an Interim Altitude - Graphic Trial Planning (Trial plan on Plans Display and Trial Plan data block on GPD )

Quit Options Next GoTo Back Section 1: Introduction Repeat

- Audio:	      Production Notes:

| In this lesson, we’ll cover some additional URET trial planning features. The lesson explains invalid trial plans, resubmitting invalid trial plans, sending an amendment message based on a trial plan, sending an interim altitude based on a trial plan, and creating a graphic trial plan in the GPD. | CU of a trial plan on the Plans Display and a Trial Plan data block on the GPD. Add bullets in sync with audio |
| --- | --- |

Branching:

| Next | 20 | Back | inactive |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 2 | Invalid Trial Plans |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 20 | Format |
| Lesson 6 |  | More Trial Planning Features |  |  |  |

Invalid Trial Plans: - Result from a change in a Current Plan that has an impact on a trial plan 511 PDT3232 150 260 CHO..DEKAY.V36.EKN..IHD.NESTO1.PIT BTA3053 2120Z BLR555 2120Z BTA3053.T1 160 AM 501 ALT 160 Plans Display NEW 1:50 Graphic Plan Display – Current Time 1 BTA3053 140C CLE 245 BTA3053.T1

Quit Options Next GoTo Back Section 2: Invalid Trial Plans Repeat

- Audio:	    Production Notes:

| When a Current Plan changes, due to a Host flight plan amendment or a reconformance, and that change has a significant operational impact on the results of a Trial Plan, an invalid trial plan will appear in the Plans Display and in the GPD. | Plans Display and GPD with invalid trial plans. Both are green. |
| --- | --- |
| In the Plans Display, an invalid trial plan is indicated by a dashed line box surrounding the affected trial plan. The box is the same color as the trial plan. | Bring the Plans display to the front and put a yellow highlight box around the dashed box. |
| On the GPD, an invalid trial plan is indicated by a dashed line box surrounding the affected trial plan ID. | Bring the GPD to the front and point to box around trial plan ID with a yellow arrow. |
| The route line for the trial plan will be dashed as well. Both the dashed box and line will be the same color as the trial plan ID. | Add a yellow arrow pointing to the dashed route line. |

Branching:

| Next | 30 | Back | 10 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 2 | Invalid Trial Plans |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 30 | Resubmitting |
| Lesson 6 |  | More Trial Planning Features |  |  |  |

To Resubmit a Trial Plan: - Middle-click on the Trial Plan ID in either the Plans Display or the GPD. Note: If you middle-click on the Trial Plan ID of an entry in the Plans Display that is NOT coded invalid, the Trial Plan and all associated problem information will be displayed on the GPD. Try It: Resubmit the invalid Trial Plan for BTA3053. 511 PDT3232 150 260 CHO..DEKAY.V36.EKN..IHD.NESTO1.PIT BTA3053 2120Z BLR555 2120Z BTA3053.T1 160 AM 501 ALT 160 Plans Display NEW 1:50

Quit Options Next GoTo Back Section 2: Invalid Trial Plans Repeat

- Audio:	    Production Notes:

| A trial plan can be resubmitted from the Plans Display and the GPD when the results of the trial have been rendered invalid. | Plans Display with invalid trial plan. Plan is green. |
| --- | --- |
| Try It. Resubmit the invalid trial plan for Jetlink thirty fifty-three. | Add bullet, note, and Try it text. |
| If middle click on BTA3053.T1: |  |
| Correct. You can also resubmit a trial plan using the Plan Options menu. | Correct feedback. Change Plans Display as shown below. |
| If middle click anywhere but BTA3053.T1: |  |
| Incorrect. To resubmit a trial plan, middle click on the entry’s trial plan ID. Try again. | Incorrect feedback. Highlight BTA3053.T1. |
| If left or right click anywhere: |  |
| No. For this task, use the MIDDLE trackball button. Try again. | Incorrect feedback. |

Branching:

| Next | 40 | Back | 20 |  |  |
| --- | --- | --- | --- | --- | --- |

BTA5053.T2, BLR555 and 2120Z are yellow

511 PDT3232                  150     260     CHO..DEKAY.V36.EKN..IHD.NESTO1.PIT

BTA3053                        2120Z

BLR555                          2120Z

BTA3053.T2                         160

BLR555                    2120Z

AM  501     ALT  160

Plans Display

NEW

2:00

| Course: |  | URET Phase 2 | Section: | 3 | Send AM & Send INT |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 40 | Send AM |
| Lesson 6 |  | More Trial Planning Features |  |  |  |

To Send an Amendment: - Left-click on the Trial Plan ID in either the Plans Display or the GPD (if it is not already highlighted) - Left-click on the SEND AM button Try It: Send an Amendment Message for BTA3053 based on its Trial Plan. Graphic Plan Display – Current Time 1 BTA3053 140C CLE 245 BTA3053.T1 Send AM

Quit Options Next GoTo Back Section 3: Send AM & Send INT Repeat

- Audio:	    Production Notes:

| Once you have created a trial plan, you can send a flight plan amendment message based on that trial plan for aircraft within and outside of the sector. You can do this from either the Plans Display or the GPD. | Plans Display with invalid trial plan. Plan is green. See old M3L2, page 10 for graphic Start with Reverse video on BTA3053.T1. |
| --- | --- |
| Try It. Send an amendment message for Jetlink thirty fifty-three, based on it’s trial plan. | Add bullets and Try it text. |
| If left- click on BTA3053.T1: |  |
| The trial plan ID is already highlighted. Clicking on it would de-select it, so you can skip this step. |  |
| If left-click on SEND AM: |  |
| Good. If you send an amendment for an aircraft outside of your sector, the eligibility menu will appear. | Correct feedback. Change GPD as shown below. Put Plans Display as shown below on top. |
| If right or middle click anywhere or left click anywhere but SEND AM: |  |
| No. You need to left click on the Send AM message. Try again. | Incorrect feedback. Highlight SEND AM button. |

Branching:

| Next | 50 | Back | 30 |  |  |
| --- | --- | --- | --- | --- | --- |

BTA3053.AM                          AM  501     ALT  160

Accept (in green text)               Accept (in white text)

Plans Display

Graphic Plan Display – Current Time

BTA3053

160     140

CLE 245

Send AM

| Course: |  | URET Phase 2 | Section: | 3 | Send AM & Send INT |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 50 | Send INT |
| Lesson 6 |  | More Trial Planning Features |  |  |  |

To Send an Interim Altitude Message: - Left-click on the Trial Plan ID in either the Plans Display or the GPD - Left-click on the SEND INT button Try It: Send an Interim Altitude Message for BTA3053 based on it’s Trial Plan. BTA3053.T1 160 AM 501 ALT 160 Plans Display 1:50 SEND INT

Quit Options Next GoTo Back Section 3: Send AM & Send INT Repeat

- Audio:	    Production Notes:

| You can also send an interim altitude message based on that trial plan. You can do this from either the Plans Display or the GPD. | Plans Display with invalid trial plan. Plan is green. See M3L2 page 16 for graphics. Reverse video on BTA3053.T2 |
| --- | --- |
| Try It. Send an interim altitude message for Jetlink thirty fifty-three, based on it’s trial plan. | Add bullets and Try it text. |
| If left- click on BTA3053.T2: |  |
| The trial plan ID is already highlighted. Clicking on it would de-select it, so you can skip this step. |  |
| If left-click on SEND INT: |  |
| Good. | Correct feedback. Change Plans Display as shown below. |
| If right or middle click anywhere or left click anywhere but SEND INT: |  |
| No. You need to left click on the Send INT message. Try again. | Incorrect feedback. Highlight SEND INT button. |

Branching:

| Next | 60 | Back | 40 |  |  |
| --- | --- | --- | --- | --- | --- |

BTA3053.AM                          INT  160  501

Accept (in green text)              Accept (in white text)

Plans Display

| Course: |  | URET Phase 2 | Section: | 4 | Graphic Trial Plan |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 60 | Altitude and Route |
| Lesson 6 |  | More Trial Planning Features |  |  |  |

To Create a Graphic Trial Plan: - Left-click on the Aircraft ID, leader line, position symbol, or trial plan ID - Left-click on the GRAPHIC button - Construct a new route by left clicking on a geographic point. - From the raised Graphic Menu, type a new altitude and press the ENTER key. - If needed, select more geographic points (and Altitudes). - When the route is completed, middle-click the trackball. - Left-click the Create TP button. . Graphic Plan Display – Current Time 1 BTA3053 140C CLE 245 Send AM Graphic Menu

Quit Options Next GoTo Back Section 4: Graphic Trial Plan Repeat

- Audio:	    Production Notes:

| You can create a trial plan graphically from the GPD using the Graphic menu. You can create graphic trial plans for altitude, route, or altitude and route together. | Plans Display. Graphic button is grayed out. See old M3L2, page 7 for graphics and animation sequence. |
| --- | --- |
| To create an altitude and route graphic trial plan, left click on the Aircraft ID, leader line, position symbol, or trial plan ID. | Animate cursor moving to BTA3053. Show BTA3053 in reverse video. Make Graphic button white. |
| Then, left click on the Graphic menu bar button. | Animate cursor moving to Graphic… Change button to Gray with white letters. |
| ## Construct a new route by left-clicking on a geographic point in the GPD. | Animate cursor changing to + and drawing new route. |
| ## If you wish to edit the new route, right-click on the route segment to delete it. |  |
| ## If you are creating an altitude and route trial plan, type a new altitude in the Graphic Menu and press ENTER. If you are creating only a route trial plan, don’t enter an altitude | Animate typing 160 in input box in menu. |
| If needed, select more geographic points and altitudes. |  |
| When the route is completed, middle-click the trackball. | Add *160 to GPD. |
| Finally, left-click the Create TP button. | Change Create TP to reverse video, remove menu from screen, add trial plan. |
| You’ll get to try the Graphic Trial plan feature in the DYSIM. |  |

Branching:

| Next | 70 | Back | 50 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 4 | Graphic Trial Plan |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 70 | Altitude Only |
| Lesson 6 |  | More Trial Planning Features |  |  |  |

To Create an Altitude Only Graphic Trial Plan: Default Point Location: - Left-click on the Aircraft ID, leader line, position symbol, or trial plan ID - Left-click on the GRAPHIC button - Left-click on the Altitude Only option - Type an altitude and press ENTER - Left-click the Create TP button. Try It: Create an Altitude Only graphic trial plan for BTA3053. Trial plan an altitude of FL190 at the default location. . Graphic Plan Display – Current Time 1 BTA3053 140C CLE 245 Send AM Graphic Menu

Quit Options Next GoTo Back Section 4: Graphic Trial Plan Repeat

- Audio:	    Production Notes:

| You can create an Altitude Only graphic trial plan for a default point location or a select point location. | Plans Display. Start with GRAPHIC button grayed out. |
| --- | --- |
| Try it. Create a graphic trial plan for Jetlink thirty fifty-three. Trial plan an altitude only of flight level one niner zero. |  |
| Step 1: left click on the Aircraft ID, leader line, position symbol for BTA3053. | If done correctly, reverse video on aircraft ID, make Graphic button white. |
| If middle or right click anywhere or left click anywhere except Aircraft ID, leader line, position symbol: |  |
| No. To create a graphic trial plan, left click on the Aircraft ID, leader line, position symbol. Try again. | Incorrect feedback. Highlight BTA3053’s data block, position symbol, leader line |
| Step 2: Left click on Graphic menu bar button. | If done correctly, bring up Graphic Menu |
| If middle or right click anywhere or left click anywhere except Graphic: |  |
| Next, you need to left click on the Graphic menu bar button. Try again. | Incorrect feedback. Highlight Graphic menu bar button. |
| ## Step 3: left click on the Altitude Only option in the Graphic Menu. | If done correctly, fill in box before Altitude Only with white. Put flashing cursor in input box. |
| ## If middle or right click anywhere or left click anywhere except Altitude Only: |  |
| ## Incorrect. The next step is to left click on the Altitude Only option. Try again. | Incorrect feedback. Highlight Altitude Only. |
| ## Step 4: Type 190 in input box and press keyboard ENTER. | If done correctly, add *190 to default point (where is this?) |
| ## If click anywhere with either trackball button: |  |
| Incorrect. Now you need to enter the altitude, in this case one niner zero, and press ENTER. | Incorrect feedback. Highlight input box. |
| If type anything other than 190: |  |
| You have entered the wrong altitude. Please enter one niner zero, and press ENTER. Try again. | Incorrect feedback. Highlight input box. |
| Step 5: Left click on Create TP. | If done correctly, reverse video on Create TP, then remove Graphic Menu. Add Trial Plan to GPD (need this). |
| Very good. If you wanted to create an altitude only graphic trial plan for a select point location, you would have Left click on a point on the route before typing in an altitude. | Correct feedback. |
| If middle or right click anywhere or left click anywhere except Create TP: |  |
| No, the last step is to left-click the Create TP button. Try again. | Incorrect feedback. Highlight Create TP. |

Branching:

| Next | 80 | Back | 60 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 5 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 80 | Introduction |
| Lesson 6 |  | More Trial Planning Features |  |  |  |

(photo of URET) # End of Lesson Exercises

Quit Options Next GoTo Back Section 5: Exercises Repeat

- Audio:	    Production Notes:

|  | Photo of URET |
| --- | --- |

Branching:

| Next | 90 | Back | 70 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 5 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 90 | Exercise 1 |
| Lesson 6 |  | More Trial Planning Features |  |  |  |

Exercise 1 of 3 Resubmit the invalid trial plan. (Place Correct and Incorrect text feedback here when appropriate) Graphic Plan Display – Current Time 1 BTA3053 140C CLE 245 BTA3053.T1

Quit Options Next GoTo Back Section 5: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | GPD with invalid trial plan. |
| --- | --- |
| If middle click on BTA3053.T1: |  |
| Correct. | Correct feedback. Change GPD: Change route line to solid yellow and remove dashed box. Auto advance. |
| If middle click anywhere but BTA3053.T1: |  |
| Incorrect. To resubmit a trial plan, middle click on the entry’s trial plan ID. Try again. | Incorrect feedback. Highlight BTA3053.T1. |
| If left or right click anywhere: |  |
| No. For this task, use the MIDDLE trackball button. Try again. | Incorrect feedback. |

Branching:

| Next | 100 | Back | 80 |  |  |
| --- | --- | --- | --- | --- | --- |

| Course: |  | URET Phase 2 | Section: | 5 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 100 | Exercise 2 |
| Lesson 6 |  | More Trial Planning Features |  |  |  |

Exercise 2 of 3 Send a flight plan amendment for STEEL71’s trial plan from the GPD. (Place Correct and Incorrect text feedback here when appropriate) Graphic Plan Display – Current Time STEEL71.T1

Quit Options Next GoTo Back Section 5: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | GPD with trial plan. See old M3L2, page 24 for graphic |
| --- | --- |
| Step 1: left- click on Steel71.T1: | If done correctly: Reverse video on STEEL71.T1 |
| If right or middle click anywhere or left click anywhere but Steel71.T1: |  |
| ## Incorrect. You need to left click on Steel seventy-one’s trial plan ID. Try again. | Incorrect feedback. Highlight STEEL71.T1 |
| Step 2: left-click on SEND AM: | If done correctly, add Eligibility menu. |
| If right or middle click anywhere or left click anywhere but SEND AM: |  |
| No. You need to left click on the Send AM message. Try again. | Incorrect feedback. Highlight SEND AM button. |
| Very good! | Correct feedback. Auto advance. |

Branching:

| Next | 110 | Back | 90 |  |  |
| --- | --- | --- | --- | --- | --- |

STEEL71.AM                          AM  256     ALT  160

Accept (in green text)               Accept (in white text)

Plans Display

| Course: |  | URET Phase 2 | Section: | 5 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 110 | Exercise 3 |
| Lesson 6 |  | More Trial Planning Features |  |  |  |

Exercise 3 of 3 Send an interim altitude for USA1729 from the Plans Display. (Place Correct and Incorrect text feedback here when appropriate) USA1729.T1 220 AM 266 ALT 220 Plans Display 1:55 SEND INT

Quit Options Next GoTo Back Section 5: End of Lesson Exercises Repeat

- Audio:	     Production Notes:

|  | GPD with trial plan. See old M3L2, page 26 for graphic |
| --- | --- |
| Step 1: left- click on USA1729.T1: | If done correctly: Reverse video on USA1729.T1 |
| If right or middle click anywhere or left click anywhere but USA1729.T1: |  |
| ## Incorrect. You need to left click on U.S. Air seventeen twenty-nine’s trial plan ID. Try again. | Incorrect feedback. Highlight USA1729.T1 |
| Step 2: left-click on SEND INT: |  |
| Good. | Correct feedback. Change Plans Display as shown below. |
| If right or middle click anywhere or left click anywhere but SEND INT: |  |
| No. You need to left click on the Send INT message. Try again. | Incorrect feedback. Highlight SEND INT button. |

Branching:

| Next | 130 | Back | 100 |  |  |
| --- | --- | --- | --- | --- | --- |

USA1729.AM                          INT   160  266

Accept (in green text)               Accept (in white text)

Plans Display

| Course: |  | URET Phase 2 | Section: | 5 | End of Lesson Exercises |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 130 | Summary |
| Lesson 6 |  | More Trial Planning Features |  |  |  |

## Exercise; Correct 1st Try?. - Resubmit a Trial Plan.. 2. Send a Flight Plan Amendment Message.. - Send an Interim Altitude Message.. Click on an exercise to review the topic.

Quit Options Next GoTo Back Section 5: End of Lesson Exercises Repeat

- Audio:	    Production Notes:

|  | Table of exercises. Clicking on an exercise name returns the CBI to the point in the lesson that covers the question. Touch area should cover entire row. |
| --- | --- |

Branching:

| Next | 140 | Back | 110 | If click Ex 1 | 30 |
| --- | --- | --- | --- | --- | --- |
| If click Ex 2 | 40 | If click Ex 3 | 50 |  |  |

| Course: |  | URET Phase 2 | Section: | 6 | Summary |
| --- | --- | --- | --- | --- | --- |
| Day 3 |  |  | Frame | 140 | Summary |
| Lesson 6 |  | More Trial Planning Features |  |  |  |

Topics Covered: - Invalid Trial Plans - Resubmitting a Trial Plan - Sending an Amendment - Sending an Interim Altitude - Graphic Trial Planning (Trial plan on Plans Display and Trial Plan data block on GPD )

Quit Options Next GoTo Back Section 6: Summary Repeat

- Audio:	    Production Notes:

| You have completed the lesson covering more trial planning features | CU of a trial plan on the Plans Display and a Trial Plan data block on the GPD. |
| --- | --- |

Branching:

| Next | EXIT | Back | 130 |  |  |
| --- | --- | --- | --- | --- | --- |